# Evaluation Assumptions

Order ID: 41000076578

Project ID: 08-17-5149 (TIP #SS-4908BK)

GPS Coordinates: 35.178191, -79.427958

County / Division: Moore County / Division 8

Study Type: Intersection Analysis with a 150' Y-line (4-Leg, signalized)

Location: SR 1309 (Morganton Road) at SR 1905 (Felton Capel Lane)/Beverly Lane, Southern Pines

Signal ID: 08-0474

Countermeasure: Install flashing yellow arrows: four-section flashing yellow arrow heads for protected-permissive left turns on all four approaches, with the signal changed from 2-phase to 8-phase operation (signal plan sealed 6/6/2017)

Statement of Problem: Left turn same road type crashes are occurring.

Project Cost: $23,000

Project Completion: 11/2/2021

The compliance memo (HNTB, 2/19/2024) gives a construction begin and completion date of 11/2/2021 and shows flashing yellow arrows on all four approaches.

November 2021 is taken as a 1 month construction period.

Time Periods:

Before Period: 3/1/2017 - 10/31/2021 (4 years, 8 months)

Construction: 11/1/2021 - 11/30/2021 (1 month)

After Period: 12/1/2021 - 7/31/2026 (4 years, 8 months)

Target Crashes: Left Turn Same Roadway Crashes on Treated Approaches (all four approaches: EB and WB SR 1309, SB SR 1905, NB Beverly Lane)

Project Dev Crash Summary: 12/1/2011 to 11/30/2016: 34 total crashes (5 B, 7 C, 22 PDO), 12 total injury crashes; 10 target injury crashes, CR = 4.1 target injury crashes per the Master Evaluation Spreadsheet.

Additional Notes:

The Master Evaluation Spreadsheet notes fatal crash M230911001 nearby. Its number points to 9/11/2023, in the after period; it will be checked against the 150' Y-line.

AADT stations: 0630000648 on SR 1309 west of the intersection, 0630000095 on SR 1309 east, and 0630000044 on SR 1905 north. Beverly Lane has no station; its volume will be estimated and rounded to the nearest hundred.

Posted speeds: 45 mph on SR 1309 and SR 1905; Beverly Lane has no posted speed (25 mph assumed on the signal plan).